Technical Specification Sheet

Remote Terminal Unit - ELIoT-RTU

Model: ELR-V1.0

## General

| Power Input | 110 V – 230 V AC |
| --- | --- |
| Power Consumption | 1.8 W |
| Enclosure Material | ABS |
| Enclosure Dimensions | 160 x 160 x 55 mm |
| Enclosure IP Rating | IP65 |

## Processor

| CPU | Xtensa® dual-core 32-bit LX7 microprocessor |
| --- | --- |
| CPU Clock Speed | 240 MHz |
| ROM | 8 MB |
| RAM | Up to 8 MB |

## Ports

| Wired Interfaces | Ethernet, RS-485, USB |
| --- | --- |
| General Purpose I/O | 4x GPIO |

## Connectivity

| Cellular | LTE Cat 1 (LTE-FDD B1/B3/B5/B7/B8/B20), up to 10 Mbps downlink / 5 Mbps uplink |
| --- | --- |
| Cellular Fallback | GSM/GPRS/EDGE, 900/1800 MHz |
| Wi-Fi | 2.4 GHz, 802.11 b/g/n |
| Bluetooth | Bluetooth 5.0 Low Energy (BLE) |
| Ethernet | 10/100 Mbps |
| Ethernet Tested Throughput | TCP: 3.78 Mbps · UDP: 3.84 Mbps |
| SIM | Nano SIM |
| Antenna | Internal / external antenna support for Wi-Fi, BLE, and LTE |

## Storage & Timekeeping

| Built-in NOR Flash | 4 MB |
| --- | --- |
| SPIFFS Partition | 2 MB |
| SD Card Slot | microSD, up to 32 GB |
| Real Time Clock | Battery-backed, network time synchronization |

## I/O & Accessories

| Relay Output | 250 V, 3 A, NO/NC |
| --- | --- |
| Isolated Input | 5 V compatible |
| Tactile Button | Yes |
| USB Port | Yes |

## Indicators

| Network Status LED | Indicates connectivity state |
| --- | --- |
| Breathing Light | System status indicator |
| Fault LED | Indicates fault condition |

## Power Supply Standards

| Safety Standard | UL1012, EN60950, UL60950 |
| --- | --- |
| Insulation Voltage | I/P–O/P: 2500 Vac |
| Insulation Resistance | I/P–O/P > 100 MΩ / 500 Vdc, 25 °C, 70% RH |
| Conduction & Radiation | EN55011, EN55022 (CISPR22) |
| Electrostatic Discharge | IEC/EN 61000-4-2, Level 4 (8 kV / 15 kV) |
| RF Radiation Immunity | IEC/EN 61000-4-3 |

## Certifications

| Cellular Module | CE, RoHS |
| --- | --- |
| Wi-Fi / Bluetooth Module | FCC, CE, IC, SRRC, Bluetooth SIG (BQB), Wi-Fi Alliance, RoHS/REACH |

## Security — Wi-Fi/Bluetooth Module (ESP32-S3), Silicon Level

| Cryptographic Accelerators | AES-128/256, SHA-1/224/256/384/512, RSA (up to 4096-bit), HMAC-SHA256, True Random Number Generator (TRNG) |
| --- | --- |
| Secure Boot | Hardware root of trust with RSA-PSS signature verification (Secure Boot V2) |
| Flash Encryption | 256-bit AES-XTS hardware encryption of external flash and PSRAM contents |
| Digital Signature Peripheral | Hardware-accelerated RSA signing; private key never exposed to software |
| Memory Protection | Hardware-enforced permission control over memory and peripheral access |
| Secure Key Storage | eFuse-based key storage with independent read/write protection |

## Security — Wi-Fi/Bluetooth Module (ESP32-S3), Firmware Level

| Secure Boot Enforcement | Verifies signed bootloader and application images before execution |
| --- | --- |
| Flash Encryption Enforcement | Encrypts firmware, credentials, and marked partitions in external flash |
| NVS Encryption | Encrypts stored credentials (e.g. Wi-Fi keys) held in non-volatile storage |
| Secure OTA Updates | Signature-verified firmware updates |
| Debug Interface Lockdown | JTAG and UART download mode can be permanently disabled via eFuse |
| TLS/SSL Stack | mbedTLS-based encrypted application-layer communication |

## Communication Protocol Security

| Wi-Fi | WPA2-Personal, WPA3-Personal, WPA2/WPA3-Enterprise (802.1X/EAP), Protected Management Frames (802.11w), Enhanced Open (OWE) |
| --- | --- |
| Bluetooth LE | LE Secure Connections (ECDH P-256 key exchange), AES-128-CCM link encryption, Passkey Entry / Numeric Comparison / Out-of-Band pairing |
| Cellular (LTE/GSM Module) | SSL/TLS (SSL3.0, TLS1.0, TLS1.1, TLS1.2) with server and mutual (client-certificate) authentication; secure protocol variants HTTPS, FTPS, MQTTS |

Manufactured by Emmanuel's Lanka (Pvt) Ltd.

Country of Design: Sri Lanka

Specifications are subject to change without notice.